AI-Powered Yoga Platform
Hardware Integration & Data Transfer

Physiological Feedback Layer — Implementation Guide

# 1. Overall Hardware → Software Flow

The wearable band collects physiological and motion data using GSR, MAX30102 and MPU6050. An ESP32 reads the sensors, performs basic preprocessing, and sends the live readings to a phone/laptop over BLE. The receiving application converts the BLE packets into structured data, timestamps them, and passes them to the sensor-fusion/ML layer.

GSR Sensor ─┐

MAX30102 ───┼──> ESP32 ──BLE──> Phone/Laptop ──> Fusion/ML ──> Dashboard

MPU6050 ────┘          │

└──> OLED (live local display)

# 2. Hardware Connections

| Device | Interface | ESP32 connection | Data |
| --- | --- | --- | --- |
| GSR | Analog | ADC pin (e.g. GPIO 34) | Skin conductance value |
| MAX30102 | I²C | SDA + SCL | Heart rate, estimated SpO₂ |
| MPU6050 | I²C | Same SDA + SCL bus | Acceleration + gyroscope |
| OLED SSD1306 | I²C | Same SDA + SCL bus | HR/GSR/status/alerts |

Important: MAX30102, MPU6050 and the SSD1306 OLED can share the ESP32 I²C bus because each device has an I²C address. The exact GPIO numbers can be changed in firmware. Check the breakout-board voltage requirements before wiring.

# 3. What Actually Happens During a Yoga Session?

1. ESP32 samples the GSR, MAX30102 and MPU6050 at defined intervals.

2. ESP32 performs basic filtering/validation and creates one compact data packet.

3. The same live values are shown on the OLED.

4. ESP32 exposes a BLE GATT service with a sensor-data characteristic.

5. The phone/laptop connects to the ESP32 and subscribes to notifications.

6. Every new BLE packet is received by the application and decoded into HR, SpO₂, GSR, acceleration and gyro values.

7. The application adds a timestamp/session ID and sends the structured data to the sensor-fusion/ML layer.

8. The fusion layer calculates a research/prototype strain score and combines it with the computer-vision pose result.

9. If an alert condition is sustained, the app displays the warning. A future version can also send a BLE command back to the ESP32.

# 4. Example Data Packet

Instead of sending separate values randomly, send one structured JSON packet at a fixed interval:

{
  "device_id": "YOGA_BAND_01",
  "timestamp": 1755760000,
  "hr": 84,
  "spo2": 98,
  "gsr": 612,
  "ax": 0.12,
  "ay": 0.91,
  "az": 0.08,
  "gx": 1.4,
  "gy": -0.7,
  "gz": 0.3
}

For a microcontroller prototype, JSON is easy to debug. Later, if bandwidth/power becomes important, the packet can be replaced with a compact binary format.

# 5. ESP32 BLE Prototype — Sender

This is a minimal implementation pattern. Replace the placeholder sensor-reading functions with the actual GSR/MAX30102/MPU6050 library calls.

#include <BLEDevice.h>
#include <BLEServer.h>
#include <BLEUtils.h>
#include <BLE2902.h>

BLECharacteristic *sensorChar;

#define SERVICE_UUID        "12345678-1234-1234-1234-123456789001"
#define CHARACTERISTIC_UUID "12345678-1234-1234-1234-123456789002"

void setup() {
  Serial.begin(115200);

  // Initialize GSR, MAX30102, MPU6050 and OLED here.

  BLEDevice::init("YogaBand");
  BLEServer *server = BLEDevice::createServer();

  BLEService *service = server->createService(SERVICE_UUID);

  sensorChar = service->createCharacteristic(
      CHARACTERISTIC_UUID,
      BLECharacteristic::PROPERTY_READ |
      BLECharacteristic::PROPERTY_NOTIFY
  );

  sensorChar->addDescriptor(new BLE2902());

  service->start();
  BLEDevice::getAdvertising()->start();
}

void loop() {
  // Replace these with real sensor readings.
  int gsr = 612;
  int hr = 84;
  int spo2 = 98;

  String packet =
      "{\"gsr\":" + String(gsr) +
      ",\"hr\":" + String(hr) +
      ",\"spo2\":" + String(spo2) + "}";

  sensorChar->setValue(packet.c_str());
  sensorChar->notify();

  // Also update OLED here.
  delay(500);
}

# 6. Receiver Side — How the Data Is Extracted

The phone/laptop application acts as the BLE receiver. It discovers the YogaBand service, subscribes to the sensor characteristic, receives each notification, parses the JSON, and forwards the values to the backend.

# Conceptual Python receiver flow

on_ble_notification(packet):
    data = json.loads(packet)

    hr   = data["hr"]
    spo2 = data["spo2"]
    gsr  = data["gsr"]

    # Add timestamp/session ID
    # Send data to fusion/ML API
    send_to_backend(data)

For the first implementation, Python can be used on a laptop for easy testing. For the final wearable/mobile version, the same BLE protocol can be implemented in an Android app.

# 7. Backend Data Flow

A clean software pipeline is:

- BLE Receiver → decode packet

- Validation/filtering → remove invalid sensor samples

- Sensor Fusion → combine HR + GSR + motion features

- ML/Rule Engine → calculate prototype strain score

- Computer Vision → provide pose/joint feedback

- Fusion of pose + physiological state → generate combined feedback

- Database → store session history

- Dashboard → show live values and progress

# 8. Recommended First Prototype

Do not build everything at once. First prove the hardware data path:

- ESP32 + GSR → Serial Monitor

- Add MAX30102 → verify HR/SpO₂

- Add MPU6050 → verify acceleration/gyro

- Add OLED → display live values

- Add BLE → send one JSON packet every 0.5–1 second

- Receive packets on laptop/phone

- Save received data as CSV/SQLite

- Only then add sensor fusion and ML

- Finally connect the computer-vision module